Name: Muhammad Sajawal

Reg No. # 24-Arid-817

Class : BSCS(4A)

# Registers

# 1. List of Registers

Below is a list of commonly used registers in the 8086 processor:

Accumulator Register (AX)

Base Register (BX)

Count Register (CX)

Data Register (DX)

Code Segment (CS)

Data Segment (DS)

Stack Segment (SS)

Extra Segment (ES)

Stack Pointer (SP)

Base Pointer (BP)

Source Index (SI)

Destination Index (DI)

Instruction Pointer (IP)

Flag Register

# 2. Additional Statement

As mentioned in the comments, registers are the fastest memory locations directly built into the CPU, used for storing temporary data, memory addresses, and instructions to speed up execution compared to accessing main memory.

# 3. Types of Registers

| Register Type | Examples |
| --- | --- |
| General Purpose Registers | AX, BX, CX, DX |
| Segment Registers | CS, DS, SS, ES |
| Pointer Registers | SP, BP, IP |
| Index Registers | SI, DI |
| Flag/Status Register | FLAGS |

# 4. Input-Output Process

The input-output process in assembly language involves moving data between the CPU registers (typically the Accumulator AL/AX) and peripheral I/O ports. The IN instruction is used for input (reading a byte/word from an I/O port into AL/AX). The OUT instruction is used for output (writing a byte/word from AL/AX to an I/O port).

# 5. Technical Examples

• Memory to Register: MOV AX, [1000H] (Loads the 16-bit value at memory offset 1000H into AX)

• Register to Register: MOV BX, AX (Copies the content of AX into BX)

• I/O Read: IN AL, 60H (Reads a byte from port 60H into AL)

# 6. Statement Example

An example of an assembly statement moving an immediate constant value into a general-purpose register for initialization.

# 7. Code Example & Git Link

One-line example: moving an immediate value into a register
MOV AX, 0005H

The corresponding code has been pushed to a Git repository.

Git Link: https://github.com/sajawal-333/RegistersPart2